EXPERIMENT- 3

Program code:

% Experiment 3: Illustrative Propagation and Frequency Reuse

clear;

clc;

close all;

% Representative bands and link-budget assumptions

freqGHz = [0.7 3.5 28];

freqMHz = freqGHz * 1000;

bandName = ["Sub-1 GHz"; "Mid-band"; "mmWave"];

txPowerdBm = 30;

gtDbi = 0;

grDbi = 0;

distanceKm = logspace(log10(0.1), log10(2), 200);

distanceAtPointKm = 0.5;

% Free-space path loss (d in km, f in MHz)

fspl = @(fMHz,dKm) 32.44 + 20*log10(fMHz) + 20*log10(dKm);

lossAtPoint = fspl(freqMHz, distanceAtPointKm);

rxAtPoint = txPowerdBm + gtDbi + grDbi - lossAtPoint;

linkTable = table(bandName, freqGHz(:), lossAtPoint(:), rxAtPoint(:), ...

'VariableNames', {'Band', 'Frequency_GHz', 'FSPL_dB', 'EstimatedRx_dBm'});

disp(linkTable);

% Path loss and received power over distance

figure;

tiledlayout(1,2);

nexttile;

hold on;

grid on;

for k = 1:numel(freqMHz)

plot(distanceKm, fspl(freqMHz(k), distanceKm), 'LineWidth', 1.5);

end

set(gca, 'XScale', 'log');

xlabel('Distance (km)');

ylabel('Path loss (dB)');

legend(bandName, 'Location', 'northwest');

title('Free-Space Path Loss');

nexttile;

hold on;

grid on;

for k = 1:numel(freqMHz)

rx = txPowerdBm + gtDbi + grDbi - fspl(freqMHz(k), distanceKm);

plot(distanceKm, rx, 'LineWidth', 1.5);

end

set(gca, 'XScale', 'log');

xlabel('Distance (km)');

ylabel('Estimated received power (dBm)');

legend(bandName, 'Location', 'southwest');

title('Illustrative Received Power');

% Simple first-tier co-channel reuse estimate

cellRadiusKm = 0.5;

reuseN = [1 3 7 12];

pathLossExponent = 3.5;

reuseDistance = sqrt(3*reuseN) * cellRadiusKm;

sirLinear = (reuseDistance/cellRadiusKm).^pathLossExponent / 6;

sirDb = 10*log10(sirLinear);

reuseTable = table(reuseN(:), reuseDistance(:), sirDb(:), ...

'VariableNames', {'ReuseClusterN', 'ApproxInterfererDistance_km', 'ApproxSIR_dB'});

disp(reuseTable);

figure;

plot(reuseN, sirDb, 'o-', 'LineWidth', 1.5);

grid on;

xlabel('Reuse cluster size N');

ylabel('Approximate SIR (dB)');

title('Simplified Co-Channel SIR versus Reuse Cluster Size');

fprintf('At %.2f km, estimated received powers are:\n', distanceAtPointKm);

disp(table(bandName, rxAtPoint(:), ...

'VariableNames', {'Band', 'Rx_dBm'}));