ASSIGNMENT-1

ECA5604 – ADVANCED WIRELESS COMMUNICATION SYSTEMS

CO1: Integrated Engineering Analysis and Planning Assignment

STAGE 1: EVOLUTION OF MOBILE COMMUNICATION SYSTEMS

1.1 Comparison of 1G to 6G

| Generation | Technology | Main features | Limitations |
| --- | --- | --- | --- |
| 1G | AMPS, analog FDMA | Analog voice communication | Poor security and limited capacity |
| 2G | GSM, digital TDMA | Digital voice, SMS, improved security | Limited data rates |
| 3G | UMTS, 5 MHz carrier | Mobile internet and video calling | Lower capacity than modern broadband systems |
| 4G | LTE, OFDMA and MIMO | High-speed internet and video streaming | Performance depends on bandwidth and interference |
| 5G | NR, beamforming, mid-band and mmWave | High data rates, low-latency applications, massive IoT | Higher-frequency coverage can be challenging |
| 6G | Future technologies such as THz and AI-native networking | Potential holographic communication, tactile internet and integrated sensing | Still under development |

1.2 Calculate Wavelength at 800 MHz

Given Data:

- Speed of light, c = 3 × 10⁸ m/s

- Frequency, f = 800 MHz = 800 × 10⁶ Hz

Formula:

Wavelength is calculated using:

λ = c / f

Substitution:

λ = (3 × 10⁸) / (800 × 10⁶)

λ = 3,000,000,000 / 800,000,000

Final Answer:

λ = 0.375 m = 37.5 cm

Explanation: The wavelength of a radio wave is the distance covered by one complete wave cycle.

1.3 Calculate the Number of 1G Channels

Given Data:

- Total frequency allocation = 10 MHz

- Bandwidth per channel = 30 kHz

Formula:

Number of channels = Total bandwidth / Bandwidth per channel

Step 1: Convert MHz into kHz.

10 MHz = 10 × 1000 = 10,000 kHz

Step 2: Substitute the values.

Number of channels = 10,000 / 30

Number of channels = 333.33

Final Answer:

Number of complete channels = 333

Practical limitation: This is an idealized result. It ignores guard bands, duplexing requirements and other practical frequency-allocation constraints. Therefore, the actual number of usable channels may be lower.

STAGE 2: CELL COVERAGE AND FREQUENCY REUSE

2.1 Calculate the Area of One Hexagonal Cell

Given Data:

- Cell radius, r = 2 km

Formula:

Area of a regular hexagonal cell:

A = (3√3 / 2) × r²

Where:

- A = area of the cell

- r = cell radius

Substitution:

A = (3 × √3 / 2) × 2²

A = (3 × 1.732 / 2) × 4

A = 10.392 km²

Final Answer:

Area of one cell = 10.39 km²

2.2 Calculate the Total Area of the 19-Cell Cluster

Given Data:

- Number of cells = 19

- Area of one cell = 10.392 km²

Formula:

Total cluster area = Number of cells × Area of one cell

Substitution:

Total cluster area = 19 × 10.392

Total cluster area = 197.448 km²

Final Answer:

Total cluster area = 197.45 km²

Explanation: This is the idealized area of the 19 hexagonal cells, assuming the cells have equal dimensions and do not overlap.

2.3 Calculate the Frequency Reuse Fraction

Given Data:

- Frequency reuse cluster size, N = 7

Formula:

Frequency reuse fraction = 1 / N

Substitution:

Frequency reuse fraction = 1 / 7

Frequency reuse fraction = 0.142857

Convert into percentage:

Frequency reuse percentage = 0.142857 × 100

Final Answer:

Frequency reuse fraction = 0.1429

Frequency reuse percentage = 14.29%

Explanation: A cluster size of seven means that the available frequency resources are divided among seven cells in the idealized reuse pattern.

2.4 Calculate the Frequency Reuse Distance

Given Data:

- Cell radius, r = 2 km

- Reuse cluster size, N = 7

Formula:

Reuse distance:

D = r√(3N)

Where:

- D = distance between co-channel cell centres

- r = cell radius

- N = reuse cluster size

Substitution:

D = 2 × √(3 × 7)

D = 2 × √21

D = 2 × 4.5826

Final Answer:

D = 9.17 km

Explanation: Cells using the same frequency are separated by this idealized distance to help control co-channel interference.

2.5 Capacity-Expansion Methods

| Method | Explanation | Main advantage |
| --- | --- | --- |
| Cell splitting | Divides a large cell into smaller cells | Increases spatial reuse and capacity |
| Sectoring | Divides a cell into directional sectors | Reduces interference from unwanted directions |
| Microcells | Smaller cells in busy streets and commercial areas | Improves local capacity |
| Picocells | Small cells in offices and shopping centres | Improves indoor and localized coverage |
| Femtocells | Very small access points, often used in homes | Improves indoor coverage |

Recommendation: The operator should initially consider sectoring at existing high-demand sites and then introduce microcells in traffic hotspots.

This can improve capacity, but the operator must also manage interference and increased handoff frequency.

STAGE 3: TRAFFIC AND CHANNEL CAPACITY

3.1 Calculate the Offered Traffic

Given Data:

- Number of subscribers, n = 1000

- Traffic per subscriber, a = 0.08 Erlang

Formula:

Total offered traffic:

A = n × a

Where:

- A = offered traffic in Erlangs

- n = number of subscribers

- a = average traffic per subscriber

Substitution:

A = 1000 × 0.08

Final Answer:

A = 80 Erlangs

Explanation: The planning area generates 80 Erlangs of offered busy-hour traffic.

3.2 Calculate Blocking Probability Using Erlang B

The Erlang B formula estimates the probability that a call will be blocked because all traffic channels are occupied.

Formula:

B(C, A) = (Aᶜ / C!) / Σ(Aᵏ / k!), for k = 0 to C

Where:

- B = blocking probability

- A = offered traffic in Erlangs

- C = number of traffic channels

- ! = factorial

- Σ = summation

For easier numerical calculation, the following recurrence relation can be used:

B₀ = 1

B꜀ = (A × B꜀₋₁) / (C + A × B꜀₋₁)

Here, the subscript c in the recurrence represents the current number of channels.

Calculate for 100 Channels

Given Data:

- Offered traffic, A = 80 Erlangs

- Number of channels, C = 100

Formula:

Blocking probability = Erlang B(100, 80)

Substitution:

Evaluating the Erlang B formula using A = 80 and C = 100 gives:

B(100, 80) ≈ 0.003992

Convert to percentage:

Blocking probability = 0.003992 × 100

Final Answer:

Blocking probability = 0.3992%

Interpretation

Required blocking probability ≤ 2%.

Calculated blocking probability = 0.3992%.

Since 0.3992% < 2%, the 100-channel system satisfies the blocking requirement under the Erlang B assumptions.

3.3 Find the Minimum Number of Channels Required

Given Data:

- Offered traffic, A = 80 Erlangs

- Maximum acceptable blocking probability = 2% = 0.02

Formula:

Find the smallest integer C satisfying:

B(C, 80) ≤ 0.02

Numerical Evaluation:

| Number of channels, C | Erlang B blocking probability | Result |
| --- | --- | --- |
| 90 | 2.6232% | Fails |
| 91 | 2.2541% | Fails |
| 92 | 1.9224% | Passes |
| 100 | 0.3992% | Passes |

Final Answer:

Minimum required channels = 92

The first channel count that satisfies the 2% target is 92.

3.4 Calculate the Number of 100-Channel Groups

Given Data:

- Minimum required channels = 92

- Capacity of one channel group = 100 channels

Formula:

Number of groups = Ceiling(Minimum required channels / Channels per group)

The ceiling function rounds a number up to the next integer when necessary.

Substitution:

Number of groups = Ceiling(92 / 100)

Number of groups = Ceiling(0.92)

Final Answer:

Number of 100-channel groups = 1

Explanation: One 100-channel group is sufficient for the calculated traffic demand under the stated assumptions. This does not prove that one physical base station can cover the entire planning area; site count depends on coverage and traffic distribution.

STAGE 4: MOBILITY AND HANDOFF

4.1 Calculate Distance Travelled During the Overlap Interval

Given Data:

- Mobile speed = 60 km/h

- Overlap-zone dwell time = 6 seconds

Formula:

Distance = Speed × Time

The units must be consistent, so first convert the speed into metres per second.

Step 1: Convert the speed.

Speed = 60 × (1000 / 3600)

Speed = 16.67 m/s

Step 2: Calculate the distance.

Distance = 16.67 × 6

Distance = 100.02 m

Final Answer:

Distance travelled ≈ 100 m

Explanation: The mobile travels approximately 100 metres during the six-second overlap interval.

4.2 Estimate Handoffs per Minute

Given Data:

- Mobile speed = 60 km/h

- Nominal route cell length = 1 km

Formula:

Handoffs per minute ≈ Distance travelled per minute / Route cell length

Step 1: Convert the speed.

Distance travelled per minute = 60 / 60

Distance travelled per minute = 1 km/min

Step 2: Calculate the handoff rate.

Handoffs per minute ≈ 1 km/min / 1 km

Final Answer:

Approximately 1 handoff per minute

Assumption: One handoff occurs each time the mobile crosses a cell boundary along the route. Actual handoff frequency depends on the network layout and radio conditions.

4.3 Estimate Simple Call-Drop Risk

Given Data:

- Target-cell unavailability = 1%

- Handoff interruption = 0.5 seconds

Assumption: A handoff fails whenever the target cell is unavailable at the moment the handoff requires it.

Formula:

Estimated handoff failure probability ≈ Target-cell unavailability probability

Substitution:

Estimated handoff failure probability ≈ 1%

Final Answer:

Simplified call-drop risk ≈ 1% per handoff

The 0.5-second interruption is the specified interruption duration, not a probability. It indicates how long the interruption may last.

Limitation: Actual call-drop probability depends on retries, overlapping coverage, handoff procedures, and other failure mechanisms. The 1% result is only a simplified estimate.

4.4 Comparison of Handoff Types

| Handoff type | Operation | Advantage | Limitation |
| --- | --- | --- | --- |
| Hard handoff | Break-before-make | Efficient resource use | May cause a brief interruption |
| Soft handoff | Multiple base stations can support the connection during transition | Can improve continuity | Uses additional radio resources |
| Softer handoff | Transition between sectors of the same site in supporting systems | Helps maintain continuity between sectors | Requires suitable radio architecture |

STAGE 5: INTERFERENCE AND SPECTRUM COMPARISON

5.1 Calculate the Carrier-to-Interference Ratio

Given Data:

- Desired signal, S = −70 dBm

- Co-channel interference, I₁ = −85 dBm

Formula:

C/I (dB) = S (dBm) − I₁ (dBm)

Substitution:

C/I = −70 − (−85)

C/I = −70 + 85

Final Answer:

C/I = 15 dB

This calculation considers only the co-channel interferer.

5.2 Calculate the Total Interference Power

Given Data:

- Co-channel interference, I₁ = −85 dBm

- Adjacent-channel interference, I₂ = −90 dBm

Important: Powers expressed in dBm cannot be added directly. First, convert each power into milliwatts.

Formula 1: Convert dBm into mW.

P(mW) = 10^[P(dBm) / 10]

Co-Channel Interference

Substitution:

I₁ = 10^(−85/10) mW

I₁ = 10^(−8.5) mW

Answer:

I₁ ≈ 3.1623 × 10⁻⁹ mW

Adjacent-Channel Interference

Substitution:

I₂ = 10^(−90/10) mW

I₂ = 10⁻⁹ mW

Answer:

I₂ = 1.0000 × 10⁻⁹ mW

Add the Interference Powers

Formula 2:

Iₜ = I₁ + I₂

Substitution:

Iₜ = (3.1623 × 10⁻⁹) + (1.0000 × 10⁻⁹) mW

Iₜ = 4.1623 × 10⁻⁹ mW

Answer:

Iₜ = 4.1623 × 10⁻⁹ mW

Convert Total Interference Back to dBm

Formula 3:

Iₜ(dBm) = 10 log₁₀[Iₜ(mW)]

Substitution:

Iₜ(dBm) = 10 log₁₀(4.1623 × 10⁻⁹)

Final Answer:

Total interference power ≈ −83.81 dBm

5.3 Calculate SINR

Given Data:

- Desired signal, S = −70 dBm

- Total interference, Iₜ = −83.81 dBm

- Required SINR = 15 dB

Assumption: Receiver noise is negligible. Therefore, this calculation gives the interference-limited SINR rather than a complete SINR that includes receiver noise.

Formula:

SINR (dB) = S (dBm) − Iₜ (dBm)

Substitution:

SINR = −70 − (−83.81)

SINR = −70 + 83.81

Final Answer:

SINR ≈ 13.81 dB

Check Whether the Target Is Met

Required SINR = 15 dB

Calculated SINR = 13.81 dB

Difference = 15 − 13.81

Difference = 1.19 dB

Conclusion: The calculated SINR is 1.19 dB below the required target. The operator should reduce interference or improve the desired signal level.

5.4 Calculate Wavelengths at Three Frequencies

Given Data:

Speed of light, c = 3 × 10⁸ m/s

Formula:

λ = c / f

A. At 900 MHz

Convert the frequency:

f = 900 × 10⁶ Hz

Substitution:

λ = (3 × 10⁸) / (900 × 10⁶)

Answer:

λ ≈ 0.3333 m

B. At 3.5 GHz

Convert the frequency:

f = 3.5 × 10⁹ Hz

Substitution:

λ = (3 × 10⁸) / (3.5 × 10⁹)

Answer:

λ ≈ 0.0857 m

C. At 28 GHz

Convert the frequency:

f = 28 × 10⁹ Hz

Substitution:

λ = (3 × 10⁸) / (28 × 10⁹)

Answer:

λ ≈ 0.0107 m

5.5 Calculate Free-Space Path Loss at 100 m

Given Data:

- Distance, d = 100 m

- Speed of light, c = 3 × 10⁸ m/s

Formula:

FSPL (dB) = 20 log₁₀(4πd / λ)

Where:

- FSPL = free-space path loss

- d = distance in metres

- λ = wavelength in metres

- π ≈ 3.1416

A. At 900 MHz

Wavelength = 0.3333 m

Substitution:

FSPL = 20 log₁₀[(4 × 3.1416 × 100) / 0.3333]

FSPL = 20 log₁₀(approximately 3770)

Final Answer:

FSPL ≈ 71.53 dB

B. At 3.5 GHz

Wavelength = 0.0857 m

Substitution:

FSPL = 20 log₁₀[(4 × 3.1416 × 100) / 0.0857]

Final Answer:

FSPL ≈ 83.32 dB

C. At 28 GHz

Wavelength = 0.0107 m

Substitution:

FSPL = 20 log₁₀[(4 × 3.1416 × 100) / 0.0107]

Final Answer:

FSPL ≈ 101.38 dB

5.6 Calculate the Path-Loss Difference

Formula:

Path-loss difference = FSPL at the higher frequency − FSPL at 900 MHz

At 3.5 GHz

Difference = 83.32 − 71.53

Answer:

Difference ≈ 11.79 dB

At 28 GHz

Difference = 101.38 − 71.53

Answer:

Difference ≈ 29.85 dB

Small differences in the last decimal place may occur because of rounding.

5.7 Spectrum Comparison Table

| Parameter | 900 MHz | 3.5 GHz | 28 GHz |
| --- | --- | --- | --- |
| Wavelength | 0.3333 m | 0.0857 m | 0.0107 m |
| FSPL at 100 m | 71.53 dB | 83.32 dB | 101.38 dB |
| Difference from 900 MHz | 0 dB | 11.79 dB | 29.85 dB |
| Typical use | Broad coverage | Coverage and capacity | Localized high capacity |
| Indoor penetration | Generally better | More limited | Often challenging |
| Deployment | Broad-area coverage | Capacity expansion | Carefully planned short-range links |

Explanation: Lower frequencies generally offer better broad-area coverage and building penetration. Higher frequencies can support high-capacity links when sufficient bandwidth and suitable antenna gain are available.

The path-loss calculation assumes ideal free-space propagation. Real deployments must also account for buildings, foliage, blockage, antenna gain, and receiver noise.

STAGE 6: INTERNATIONAL SPECTRUM COORDINATION

This stage is primarily explanatory and does not require a numerical calculation.

6.1 Role of ITU-R

The International Telecommunication Union Radiocommunication Sector (ITU-R) develops recommendations and coordinates international radio-frequency spectrum use.

Its activities support efficient spectrum utilization and help reduce harmful interference between radio services.

6.2 Role of the World Radiocommunication Conference

The World Radiocommunication Conference (WRC) reviews and revises the international Radio Regulations when required.

Its decisions can affect international frequency allocations and the conditions under which different radio services operate.

6.3 Role of the Radiocommunication Bureau

The Radiocommunication Bureau (BR) supports the implementation of the Radio Regulations, processes frequency assignments, maintains international frequency records, and assists with certain interference-related matters.

6.4 Effect on the Regional Operator

The operator must check national frequency allocations, licences, permitted transmit power, emission limits, and interference-protection requirements before deploying a network.

International coordination helps countries manage spectrum consistently, but the operator must still comply with its national regulator's rules.

7. FINAL NETWORK DEPLOYMENT RECOMMENDATION

The regional operator should follow a phased strategy.

Phase 1: Broad coverage

Use suitable lower-frequency spectrum, such as 900 MHz where available and licensed, to establish broad-area coverage.

Phase 2: Capacity planning

The offered traffic is 80 Erlangs. The 100-channel group produces an estimated blocking probability of 0.3992%, which meets the 2% requirement under the Erlang B model.

Phase 3: Capacity expansion

Introduce sectoring at existing high-demand sites and deploy microcells where traffic demand is concentrated. Consider 3.5 GHz where suitable spectrum is available.

Phase 4: Interference reduction

The calculated interference-limited SINR is 13.81 dB, below the required 15 dB. Optimize frequency planning, antenna orientation, antenna tilt, and transmit power.

Phase 5: Mobility optimization

Maintain sufficient cell overlap and optimize handoff procedures. At 60 km/h, the mobile travels approximately 100 m during the six-second overlap interval.

Phase 6: Localized high-capacity services

Consider 28 GHz for suitable high-demand locations, using directional antennas and beamforming where appropriate.

Before implementation, verify the calculations against actual traffic measurements, coverage predictions, interference measurements, handoff statistics, and national spectrum regulations.

8. FINAL RESULTS SUMMARY

| Calculation | Final result |
| --- | --- |
| Wavelength at 800 MHz | 0.375 m |
| Idealized 1G channel count | 333 channels |
| Area of one hexagonal cell | 10.39 km² |
| Area of 19-cell cluster | 197.45 km² |
| Frequency reuse fraction | 0.1429 or 14.29% |
| Reuse distance | 9.17 km |
| Offered traffic | 80 Erlangs |
| Blocking probability with 100 channels | 0.3992% |
| Minimum channels for ≤ 2% blocking | 92 |
| Number of 100-channel groups | 1 |
| Distance travelled in six seconds | 100 m |
| Estimated handoffs per minute | 1 |
| Simplified call-drop risk | 1% per handoff under the stated assumption |
| Co-channel C/I | 15 dB |
| Total interference power | −83.81 dBm |
| Interference-limited SINR | 13.81 dB |
| FSPL at 900 MHz | 71.53 dB |
| FSPL at 3.5 GHz | 83.32 dB |
| FSPL at 28 GHz | 101.38 dB |

Overall Conclusion

The calculations indicate that the initial 100-channel group satisfies the traffic-blocking requirement under the Erlang B model. However, the calculated interference-limited SINR is below the required 15 dB target, so interference optimization is a priority.

A suitable deployment strategy combines broad-area coverage, targeted capacity expansion, handoff optimization, and careful spectrum planning. The final design should be validated using realistic propagation models, measured traffic demand, and actual network performance data.